Task-Aware Object Selection Accelerator: Hardware-Software Co-Design for Task-Conditioned Edge Inference on FPGA

Karthik M, N A Akilan, Devaram A

TESTBENCHERS, Submission #442

Department of Electronics and Communication Engineering, Chennai Institute of Technology, Chennai, India

- Table of Contents

Include the name of each section/subsection and the page number.

Abstract	4

I.	Introduction	5

II.	Background Research	6

III.	Goal and Objectives	7

IV.	Design Process	8

i.	Problem Statement	8

ii.	Functional Specification	8

iii.	Proposed Design	8

iv.	Analysis of Final Design	8

v.	Testing and Implementation of Final Design	8

V.	Results and Discussion	9

VI.	Conclusion	10

VII.	References	10

- List of Tables

Includes table number and captions along with page number.

- List Of Figures

< Includes figure number and captions along with page number.>

- List of Symbols

Includes all symbols along with their names.

# Abstract

This work presents a task-aware object selection accelerator designed for edge deployment on the Genesys 2 FPGA board (Xilinx Kintex-7 xc7k325tffg900-2). Conventional object detection systems are task-agnostic, detecting all visible objects with equal priority. We address this limitation by implementing a task-driven ranking pipeline that, given an image and one of 14 natural-language task descriptions (e.g., "cut something", "sit on something"), detects, ranks, and selects the single most relevant object for the given task. The software stages run on the VEGA RV64GC RISC-V processor: YOLOv8n (nano, ~3.2M parameters) performs object detection; ResNet18 extracts 128-dim per-object visual features; a task-conditioned scene descriptor computes a task-gated context vector using a learned gating network; and precomputed MiniLM (all-MiniLM-L6-v2) embeddings provide 384-dim semantic task representations with zero runtime transformer overhead. The core ranking module -- a two-layer MLP (FC1: 731->128 with ReLU, FC2: 128->1) operating on a fused 731-dimensional feature vector -- is implemented as a pipelined HLS kernel on the FPGA, achieving 731 cycles per object at 200 MHz (3.65 us/object). For a full batch of 14 objects, the FPGA kernel completes ranking in 51.2 us, approximately 10x faster than a CPU-only path. Post-implementation results on xc7k325t show 9.5% LUT, 8.8% FF, 16.7% DSP, and 14.9% BRAM utilization for the complete block design. The system handles all 14 COCO-Tasks from the reference paper and is designed for final deployment with the VEGA processor on the Genesys 2 platform.

# Introduction

Object detection is a foundational capability in computer vision, but detecting objects is only half the problem for task-oriented autonomous systems. A robotic arm attempting to "make food" must prioritize a knife or a bowl over background furniture. A visual assistance system asked to "read something" must surface a book or a sign rather than irrelevant objects. Conventional detectors -- YOLO, SSD, Faster-RCNN -- are task-agnostic: they assign confidence scores based on object category and detection confidence, not on relevance to the current human task. This leaves the entire task-selection problem unresolved at the detection stage.

This project implements a task-aware object selection pipeline directly motivated by the formulation of Sawatzky et al. ("What Object Should I Use? -- Task Driven Object Detection", CVPR 2019). Given an image and one of 14 predefined task descriptions, the system outputs the single object most relevant to that task, together with its bounding box, class label, and a numerical task relevance score.

The significance of this work lies in its edge-deployable hardware-software co-design. Prior work on task-conditioned inference typically relies on graph neural networks or large language models requiring GPU-class compute. Our system replaces the reference paper's GGNN with a two-layer MLP whose weights are quantized to 8-bit fixed-point and mapped entirely to FPGA BRAM, enabling deterministic low-latency inference on the Genesys 2 board. The final deployment target integrates the VEGA RV64GC RISC-V soft processor (CDAC Trivandrum) as the host CPU alongside the Xilinx Kintex-7 xc7k325t FPGA for hardware-accelerated ranking. This combination demonstrates a viable path for task-aware object selection at the edge -- a capability that must merge visual understanding, language grounding, and real-time compute constraints simultaneously.

# Background Research

COCO-Tasks Dataset and Reference System:
Sawatzky et al. (CVPR 2019) introduced the COCO-Tasks benchmark with 14 everyday task categories (cut, sit, hold, make food, look through, play music, read, write, scoop, pound, support, wear, carry, open/close) annotated over MS-COCO images. The reference system used a Graph Gated Neural Network (GGNN) for object-task relevance scoring, but GGNN requires iterative GPU-class message-passing over dynamic scene graphs -- infeasible on embedded processors.

Object Detection -- YOLOv8n:
YOLOv8n (Ultralytics, 2023) is a single-stage detector with ~3.2M parameters, capable of running at ~15 ms/frame on a CPU-class processor. Its compact architecture and ONNX export path make it suitable for deployment on VEGA RV64GC without GPU acceleration. Confidence threshold 0.25, max 20 detections per image.

Visual Feature Extraction -- ResNet18:
ResNet18 (He et al., 2016) is a residual network with 18 layers producing a 512-dim global average-pooled feature representation. Used as a frozen backbone with a learned Linear(512->128) projection layer, it provides compact, discriminative per-object embeddings at low inference cost. The deterministic projector initialization (torch.manual_seed(42)) ensures consistency between training and hardware deployment.

Task Encoding -- Sentence Transformers:
The all-MiniLM-L6-v2 sentence transformer (Wang et al., 2020) maps natural-language task descriptions to 384-dim dense embeddings. By precomputing embeddings offline for all 14 tasks, the production inference path requires no transformer execution: a 14x384 lookup table (5.3 KB) replaces the model entirely.

FPGA-Based Neural Network Acceleration:
Vitis HLS (Xilinx/AMD, UG1399) enables high-level synthesis of neural network layers from C++ to RTL. Key techniques used in this work include: ap_fixed<W,I> fixed-point types for weight/activation quantization, HLS DATAFLOW for pipeline-level parallelism between read/compute/write stages, HLS PIPELINE II=1 for throughput-optimized inner loops, ARRAY_PARTITION for register-file style unrolled weight arrays, and AXI-Stream interfaces for streaming data to/from the host DMA controller. Per-layer dynamic fixed-point scaling recovers effective precision by ensuring quantized weights span the full representable range.

# Goal and Objectives

Goal:
Design and implement a complete task-aware object selection system that handles all 14 COCO-Tasks and is deployable on the Genesys 2 FPGA board with the VEGA RV64GC RISC-V processor, achieving real-time inference performance with efficient hardware utilization.

Objectives:

1. Implement a full software pipeline integrating YOLOv8n object detection, ResNet18 visual feature extraction, task-conditioned scene gating, and precomputed task embedding lookup on the host VEGA processor.

2. Design a pipelined hardware accelerator for the two-layer ranking MLP (731->128->1) using Vitis HLS 2024.2, targeting the Xilinx Kintex-7 xc7k325t FPGA with AXI-Stream input/output and AXI-Lite control interfaces.

3. Achieve deterministic FPGA ranking latency below 100 us for a batch of up to 14 detected objects at a 200 MHz clock using a DATAFLOW pipeline with II = 731 cycles/object.

4. Maintain efficient FPGA resource utilization below 30% for LUT, DSP, and BRAM on the xc7k325t device, preserving headroom for VEGA processor integration and system peripherals.

5. Support all 14 COCO-Tasks through a unified inference path, with flexible task input as either natural-language text (matched offline) or direct integer ID (0-13).

6. Integrate hardware performance counters (perf_total_cycles, perf_object_count) accessible over AXI-Lite to enable runtime monitoring and adaptive batching by the host processor.

# Design Process

This section must contain the exact steps of the design process. Each step should have its own sub-section. It must contain the following sections:

## Problem Statement

Given an input image and one of 14 predefined task descriptions, the system must:

(a) Detect all objects present in the image using a standard COCO-trained object detector.

(b) Represent each detected object as a fused feature vector encoding visual appearance (what it looks like), scene context (how it relates to other objects given the task), and task semantics (what the task requires) in a single 731-dimensional vector.

(c) Score each candidate object's relevance to the given task using a trained ranking model implemented as a hardware-accelerated MLP on the FPGA.

(d) Select and return the single highest-scoring object -- its bounding box coordinates, COCO class label, and numerical relevance score.

The 14 COCO-Tasks supported are: cut something, sit on something, hold something, make food, look through something, play music, read something, write something, scoop something, pound something, support something, wear something, carry/pick up something, and open/close something.

Constraints: The complete solution must run on the Genesys 2 board (Xilinx Kintex-7 xc7k325t + VEGA RV64GC processor) without GPU-class compute. The ranking accelerator must achieve deterministic low-latency inference suitable for real-time operation.

## Functional Specification

System Inputs:
  - Image: COCO-resolution image (JPEG/PNG), pre-processed to 640x640 by YOLOv8n
  - Task: Natural-language string (e.g., "cut something") OR integer task ID (0-13)

System Outputs:
  - Best object bounding box: [x1, y1, x2, y2] pixel coordinates
  - Object class name: COCO category label (e.g., "knife", "chair")
  - Task relevance score: floating-point scalar (higher = more task-relevant)

Processing Pipeline Stages:

  Stage 1 -- YOLOv8n Detection:
  Detects up to N=20 objects with confidence > 0.25. Outputs per-object: boxes [N,4] (xyxy coordinates), scores [N], class_ids [N].

  Stage 2 -- ResNet18 Feature Extraction:
  Each detected region is cropped from the image, resized to 224x224, normalized with ImageNet statistics, and passed through frozen ResNet18 backbone + Linear(512->128) projector. Output: object_features [N, 128].

  Stage 3 -- Task-Gated Scene Description:
  Computes scene context vector [128] via task-conditioned attention pooling:
  gate_i = sigmoid(W_gate . concat(f_i, task_embed))   [learned gating]
  scene  = sum(gate_i * f_i) / sum(gate_i)             [weighted mean pooling]

  Stage 4 -- Task Embedding Lookup:
  Retrieves precomputed 384-dim MiniLM embedding for the given task from tasks.npy [14, 384]. Zero transformer latency at runtime.

  Stage 5 -- Feature Assembly:
  Concatenates per-object: [object_feat(128) | scene(128) | task_embed(384) | class_onehot(91)] -> 731-dim input vector. Full matrix [N, 731] quantized to int16 (scale x1024) for DMA.

  Stage 6 -- FPGA Ranking Accelerator (ranking_mlp_top):
  HLS kernel receives N x 731 int16 AXI-Stream beats. Computes FC1 (731->128, ReLU) + FC2 (128->1) for each object in a DATAFLOW pipeline. Returns N int16 scores via output stream.

  Stage 7 -- Argmax Selection:
  Host identifies maximum score, returns corresponding object's box, class, and score.

FPGA Hardware Interface:
  - AXI-Stream IN  (input_stream):  N x 731 x 2 bytes at 200 MHz
  - AXI-Stream OUT (output_stream): N x 2 bytes at 200 MHz
  - AXI-Lite CTRL  (s_axi_control): ap_start, ap_done, task_id(4-bit), n_objects(16-bit),
                                     perf_total_cycles(32-bit), perf_object_count(16-bit)

## Proposed Design

The proposed system is a hardware-software co-design with three principal components:

--- 1. Software Pipeline (CPU/VEGA Side) ---

The software pipeline runs on the VEGA RV64GC host processor (RV64GC ISA, up to 100 MHz, DDR3 via MIG) and implements Stages 1 through 5. YOLOv8n is exported to ONNX for RISC-V deployment. ResNet18 (ImageNet-pretrained, frozen backbone) projects each detected object region to a 128-dim visual embedding via a deterministically initialized Linear(512->128) layer (seed=42). The task-conditioned scene descriptor introduces the key architectural novelty: a trained Linear (128+384, 1) gate module (W_gate) weights each object's contribution to the scene context vector based on both its visual features and the active task embedding. This allows the scene summary to shift toward task-relevant objects -- glasses upweighted for "look through something", spoons for "scoop something". The 14 x 384 precomputed task embedding table (5.3 KB) is loaded at startup and served with zero inference overhead. The assembled [N, 731] float32 input matrix is quantized to int16 (multiply by 1024, clip to [-32768, 32767]) and transferred to the FPGA via AXI-DMA.

--- 2. FPGA Accelerator: ranking_mlp_top HLS Kernel ---

The HLS kernel is a pipelined three-stage DATAFLOW loop over N objects:

  Stage A: read_input()
  PIPELINE II=1. Drains 731 AXI-Stream beat (16-bit TDATA each) into local array input_vec[731].

  Stage B: compute_fc1()
  Systolic MAC array. fc1_w[128][731] ROM is ARRAY_PARTITION complete on dim=1 (128 copies). The j-loop (j = 0..730) is PIPELINE II=1; the inner i-loop (i = 0..127) is fully UNROLL-ed, firing 128 parallel MACs per cycle. Accumulator acc[i] += fc1_w[i][j] * input_vec[j] for each cycle. After 731 cycles, ReLU is applied: hidden[i] = max(0, acc[i] + fc1_b[i]). Accumulator type: ap_fixed<40,20> ensures no overflow for worst-case sum 731 x 2.0 x 32.0 = 46,784.

  Stage C: compute_fc2_write()
  128-element dot product: score = sum(fc2_w[i] * hidden[i]) + fc2_b. Score written as one int16 AXI-Stream beat (TLAST=1 after last object).

Type System:
  weight_t = ap_fixed<8,2>   (8-bit, 6 frac bits, range [-2.0, +1.984])
  input_t  = ap_fixed<16,6>  (16-bit, 10 frac bits, range [-32.0, +31.999])
  acc_t    = ap_fixed<40,20> (overflow-safe accumulator)

Key Novelties:
  N1. Task-conditioned scene gating: learned W_gate produces task-specific scene context, enabling the pipeline to emphasize task-relevant objects at the feature-assembly stage.
  N2. Per-layer dynamic fixed-point quantization: per-layer scale factors maximize effective precision of ap_fixed<8,2> weights (~4-bit improvement over static scaling).
  N3. Hardware performance counters: perf_total_cycles and perf_object_count AXI-Lite registers allow the host to adaptively tune n_objects to meet latency targets.
  N4. Fused 731-dim input: visual (128) + scene context (128) + task semantics (384) + structural (91 COCO one-hot) features processed in a single fixed-point forward pass.

--- 3. Genesys 2 Block Design (xc7k325tffg900-2) ---

CLK_WIZ_0         : 200 MHz LVDS differential input (AD12/AD11) -> 200 MHz BUFG
MicroBlaze         : 32-bit softcore (placeholder until VEGA IP received from CDAC)
AXI DMA            : MM2S + S2MM, 16-bit streams, connected via SmartConnect
ranking_mlp_top_0  : HLS IP at AXI-Lite base 0x40010000
TX BRAM (4 KB)     : 0x80000000 -- input vector buffer for DMA MM2S
RX BRAM (1 KB)     : 0x80010000 -- output score buffer for DMA S2MM
AXI UARTLite       : 0x40020000, FTDI FT232 (Y23/Y20), 9600 baud

Resource Utilization on xc7k325t (post-implementation, proxy build on xc7k160t):

  Resource  |  Used   | Available | Utilization
  ----------+---------+-----------+------------
  LUT       |  19,387 |   203,800 |   9.5%
  FF        |  36,009 |   407,600 |   8.8%
  BRAM 36K  |    66.5 |       445 |  14.9%
  DSP48E1   |     140 |       840 |  16.7%
  MMCM      |       1 |        --  |    --

FPGA Throughput Summary:
  - Per-object:   731 cycles / 200 MHz = 3.65 us
  - 14 objects:  10,234 cycles = 51.2 us (kernel only)
  - End-to-end:  ~130 us (including DMA setup and Python overhead)
  - vs CPU-only: ~500-800 us -> ~10-12x speedup for ranking stage

Toolchain: Vitis HLS 2024.2, Vivado 2024.2, Python 3.8+, PyTorch, Ultralytics YOLOv8, PYNQ.

## Analysis of Final Design

## Testing and Implementation of Final Design

# Results and Discussion

Present your results, draw meaningful inferences and suggest improvements and ideas about what you could have done differently. Make suggestions for future improvements. Cover aspects of your project that your team did not have the time or resources to accomplish.

# Conclusion

Provide a general overview of the work and summarize the significant contributions.

# References

This section will contain every reference that you cited in your report. Each reference must contain the authors, title, publisher, and publication date.